HEIMDALL CIRCUIT:

WIRING DIAGRAM:

CONNECTING THE RASPBERRY PI 4 WITH THE RPI CAMERA MODULE V2:

PIN DIAGRAMS/ PARTS EXPLANATION DIAGRAMS:

Raspberry Pi 4:

Infrared Thermometer MLX90614:

Servo Motor:

IR Proximity Sensor:

DC Water Pump:

128X64 LCD Graphic Display:

Relay:

Adjustable Breadboard Power Supply:

CIRCUIT INSTRUCTIONS:

- Collect all electronic components and parts required for the circuit.

- Connect the RPi Camera Module V2 to the Raspberry Pi 4.

- Begin with powering up the Raspberry Pi by connecting it to a 5V DC source, through a USB Cable.

- The Servo motor has 3 wires, of which the black wire is connected to the ground, Red to the 5V Pin and Yellow to a GPIO pin of the Raspberry Pi 4.

- The IR Proximity sensor has 3 pins. Connect the GND and Out pins to the ground of the RPi 4. Connect Vcc ( and enable, if sensor has 4 pins) to the 5V Pin of the Raspberry Pi.

- To connect the MLX90614 Infrared thermometer with the Raspberry Pi, first connect a capacitor between the Vdd and Vss terminals of the IR Thermometer.

- From MLX90614 to Raspberry Pi 4, connect Vdd to 3.3v, Vss to GND, SCL/VZ to GPIO3, and PWM/SDA to GPIO2.

- To connect the DC Submersible water pump ( DC motor in the wiring diagram) with the Raspberry Pi 4, a relay is used.

- The Relay has 6 terminals. Connect one terminal of the dc pump to the NO of relay and the other terminal of the dc pump to the GND pin of relay.

- The remaining pins of the relay, the Signal pin is connected to a GPIO pin, C pin to a 12V supply and the IN pin to the GND of the Raspberry Pi 4.

- An adjustable breadboard supply is used for the 12v DC supply. 12V is drawn from Vout+ to the circuit, GND is grounded and the AC supply is provided at Vin+ and Vin- pins.

- Connecting the 128x64 Graphic LCD display with the Raspberry Pi: Ground the Vss, CS1 and K- pins of the display.

- Connect Vdd, D/I, and A+ of the display to the 3.3V Supply Pin of Raspberry Pi.

- Connect the Vo pin of the Graphic LCD display, to a variable resistance (set to 10k ohms). With one end of the resistor grounded and the other connected to the 3.3V Supply Pin of Raspberry Pi.

- R/W and Enable of the 128x64 Graphic LCD is connected to the GPIO9 and GPIO10 pins of the Raspberry Pi.